How to Complete Your Own Risk Assessment

The risk assessment attached is only for guidance and should be adjusted to match your own operations and environment. The risk assessment should be completed as follows..

1.	Identify the hazards

2.	Determine who might be harmed and how

3.	Evaluate the risks and take precautions

4.	Record your findings

5.	Review your assessment and update if necessary

Hazards Identified	Identify all the hazards specific to your environment, the loads you 			are carrying and capability of the people.

Potential Outcome	This rates the severity of the injury that could result.

Tthis will vary based on the load you are carrying, a 250Kg safe falling on your foot will cause a more severe injury than 5 boxes of crisps.

1 – No injury

3 – Minor injury (small cuts, abrasions, bruises)

5 – Lost Time Injury (The person cannot continue working)

8 – Severe Injury (A permanent or long lasting injury)

10 – Fatal

Control Measures	What factors are already in place to mitigate the hazard 	and/or reduce the risk of injury.	Typical Control Measures are Training, Signage, PPE, Safety Barriers

Likelihood	This rates the likelihood of the hazard occurring, and is based on your own knowledge of the job and the control measures in place.

10  – Certain/Imminent

9 – Very Likely

5 – Likely

2 – Unlikely

1 – Remote

Score			Multiply the Potential Outcome x Likelihood to obtain the score. The 			greater the number the greater the risk.

Rating			Rates the remaining risk.

50-100	High – Control measures should be put in place ASAP

20-49	Medium- If possible, control measures should be found.

1-19	Low – Continue to look for improvements.

| FOR GUIDANCE ONLY | SUBJECT General Safe Use of a Sack Truck |
| --- | --- |
| ASSESSMENT DATE 18/02/2025 | ASSESSOR |
| TASK ASSESSED Recommended General Safe Use of a Sack Truck |  |
| PERSONS AFFECTED Employee X Visitor X Contractor X Public |  |
| SPECIAL GROUPS (Where individual assessments will be required.) Young Persons Nursing/Expectant Mothers Disabled |  |

| Hazards Identified | Potential Outcome | Control Measures in Place | Like- lihood | Score | Rating |
| --- | --- | --- | --- | --- | --- |
| Structural failure of the sack truck due to exceeding the Uniform Distributed Load Limit (UDL) marked on the unit. | 8 | The maximum UDL capacity as stated must not be exceeded., Best practice is the load should not be mor than 75% of the units capacity Only trained, competent individuals to use the unit. | 2 | 16 | Low |
| Operator extremities crush injury from falling loads | 8 | Loads must always be secured to the truck using a suitable strap, rated to the appropriate Uniform Distributed Load Limit (UDL) Safety footwear should always be worn transporting loads. | 2 | 16 | Low |
| Musculoskeletal strain due to the operator exceeding their personal capabilities / poor technique. | 5 | Only competent / trained individuals are to move loads using a sack truck. Task based risk assessment to be conducted prior to the commencement of the task. Task specific method statement to be followed to ensure correct use. | 2 | 10 | Low |
| Falling loads due to instability as a result of poor ground conditions | 8 | Specific task based risk assessment to be conducted prior to moving loads on uneven / unsuitable ground Loads are to always be securely fixed to the sack truck prior to commencement of the task. | 2 | 16 | Low |
| Loss of control of the unit due to tyre failure / puncture. | 8 | Pre work checks to be performed prior to commencing the task. Where Pneumatic (PN) type tyres are fitted ensure that they are inflated to the stated pressure. | 1 | 8 | Low |
| Accidental load slipping from toe plate. | 5 | Ensure that the toe plate fitted is of a suitable size and strength. To ensure stability it is recommended that the foot plate is 2/3 of the width of the load and at least 2/3 of the overall length of the load. Ensure the load is always securely strapped. | 2 | 10 | Low |
| Falling loads as a result of sack truck instability due to irregular distributed load. | 5 | To ensure stability it is recommended that the foot plate is 2/3 of the width of the load and at least 2/3 of the overall length of the load. Footplate not to be overloaded and or poor positioning of the load. Load to always be securely attached to the sack truck with an appropriately rated strap | 2 | 10 | Low |
| Structural failure of the sack truck due to loose fittings. | 5 | Daily prework checks to be conducted prior to use. Regular preventative maintenance inspections to be conducted at suitable intervals. | 2 | 10 | Low |

| 10 | 8 | 5 | 3 | 1 |  | 10 | 8 | 5 | 2 | 1 |  | High Medium Low | 50-100 20-49 1-19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fatality | Severe Injury | Lost Time Injury | Minor Injury | No Injury |  | Certain/ Imminent | Very Likely | Likely | Unlikely | Remote |  |  |  |

| Action required (note any temporary action control measures reqd) |  | Action Review Date | Completed By (Name & signature) |  |
| --- | --- | --- | --- | --- |
| Further actions that may require long term consideration |  | Action Review Date | Completed By (Name & signature) |  |
| If any issues are outstanding from “Action Review” date, detail the reasons. |  |  |  |  |
| Assessment Review Date: (Maximum annually) | 18/02/2026. | Assessment Review Date: (Maximum annually) |  |  |
| New Risk Assessment Required: |  | New Risk Assessment Required: |  |  |
| Completed by (Name): | Joe Blogs | Completed by (Name): |  |  |
| Position: | H&S Manager | Position: |  |  |
| Signature: |  | Signature: |  |  |